TL-101 Room Thermostat

Product Introduction
	[image: image1.jpg]



	TL-101 room thermostat are used to control room temperature in commercial, industrial and residential

environment via controlling fan coil units, motorized valve, motorized damper, heating pump and so on.

It uses NTC to detect the room  temperature and compares it with the set-point so as to automatically keep the room temperature stable. 

Its simple structure, reliable design & easy installation.


	
	


Part No.

[image: image2.emf]Fan3-speed Control Capability

Heat Cool Manual Damper

2-pipe

Fan coil

4-pipe

Fan

coil

2-wire

Motorized

valve

3-wire

Motorized

valve

Cycling Fan

TL-101Y

√ √

TL-101DA

√ √ √ √ √

TL-101DB

√ √ √ √ √ √

TL-101FCV2

√ √ √ √ √

Control Model  Application

Modle 


Note:

●TL-101Y can only control damper to open and close.

●When the temperature reaches the set-point, thermostat will close the valve but the fan speed will be still running for DA model,thermostat will close the valve and stop the fan running for DB model.

Available for                                                                  
●Two-pipe Fan coil unit

●Four-pipe Fan coil unit
●Motorized valve

●Air-damper
Technical Data

●Power Voltage: AC24V/AC110V/ AC210~230V, 50/60HZ. (Optional)       
●Electrical Rating<3A
● Accuracy: ±1ºC.       
●Set-point Range: 5ºC - 37ºC.
●Power Consumption: <1.5W.             
●Sensor: NTC. 
●Relative Humidity :< 92 %( no condensing)
●Dimension: 124*78*34mm (H*W*D).
Features                                                                       
●Elegant shape with high reliability

●International used LCD screen display: shows easy to operate. 

●Manual switch to select work state
●Thermostats can be directly mounted on the wall or standard wiring box.
Fuction
●COOL/ HEAT/OFF select by switch .

●Fan speed selected by switch. Fan speed: automatic, high, medium and low.
●Detect room temperature every 10seconds and show accuracy on LCD screen

●Memory mode to reset to previous conditions. (Optional).

If the power occurs to be cut off suddenly, the thermostats could be continuing to work by order if the power returns. The function is popular at Hotel, System House etc.
●Low Temperature Protection: auto start at temperature below 5℃, prevent frostbite.
CERTIFICATION:  CE   RoHS        WARRANTY: One  YEAR WARRANTY

